Technical Information Document

COE

Servers

| Document Title: | How to install manually NEC IPMI driver for BMC (NECRAS.SYS) ? |
| --- | --- |
| Document ID: | TID0404271128 |
| Creation Date: | 27 April 2004 |
| Modified Date: | 24 Novembre 2008 |
| Document Revision: | 1 |
| Product Class: | Servers |
| Product and Version: | Any server with ESMPRO/Dianascope Agent for Windows |

Symptoms / Facts

ESMPro//Dianascope Agent requires NECRAS.SYS to install.

Cause

NECRAS.SYS is the NEC IPMI driver to communicate with BMC.

This driver is mounted either when

- When Windows is installed by ExpressBuilder

- When NEC Update software is applied

But those two solutions are applicable only with Western Europe languages.

Solutions / Fixes

There is a manual way to install this driver.

Follow the instructions (ExpressBuilder CD associated to server is required):

- Once Windows installation is finished, create a folder C:\TEMP\

- Copy from ExpressBuilder NECRAS.SYS and NECRAS.INF files to C:\TEMP\

Note: 	For Windows2000, source files are located into: \WINNT\W2K\RAS

For Windows2003, source files are located into: \WINNT\DOTNET\RAS_A

- Copy from ExpressBuilder RASINST.EXE files to C:\TEMP\

Note: 	For Windows2000, source file is located into: \WINNT\W2K

For Windows2003, source file is located into: \WINNT\DOTNET

- Open a command line session, and make C:\TEMP\ as current directory

- Type the following command

## RASINST.EXE c:\TEMP\necras.inf root\necras

- In order to check installation finished successfully, open device manager and enable the display of hidden devices:

7) Open the category “Non-Plug and Play Drivers”, if the installation is successful, the line “NEC Baseboard management controller” should appear:

| Document owner: | COE Servers |
| --- | --- |
| Applies to: | Any Server with ESMPRO//Dianascope Agent for Windows |