Technical Information Document

COE

Servers

| Document Title: | Unable to boot a R440 or R460 with SAS controller |
| --- | --- |
| Document ID: | TID0703191431 |
| Creation Date: | 19 Mar. 07 |
| Modified Date: |  |
| Document Revision: | 0 |
| Product Class: | Servers |
| Product and Version: | R440 & R460 |

Symptoms / Facts

The machine is locked on POST on LSI Megaraid RAID SAS BIOS on board controller initialisation with the following message:

“Bringing up the Controller. Please wait…”

Cause

The jumpers are not set correctly on the motherboard.

Solutions / Fixes

- Power off the machine

- Unplug the power cable

- Set correctly the jumper RoMB for RAID configuration

  - jumper number 12 on “Server Hardware Quick Reference” sticker (on the small front top cover FAN)

  - This jumper will enabled or disabled the RAID on board feature depending on the customer requirement

RAID

disable

RAID

enable

-	Set correctly the jumper number 6 on “Server Hardware Quick Reference” sticker (on the small front top cover FAN). This jumper must be set in “Factory default” mode 2-3. The other position must never be used.

Never used

Factory Default

- Power On the machine

- The issue should be solved. Please do not forget to update BIOS, BMC and SDR using the last BOOT_CD.

See also

The following picture will show you all the jumper location and function for R460 and R440.

Non-Raid

Raid

Only SAS version

Normal

Clear CMOS

Normal

Clear PWD

Fan Normal

Fan Redundant

In R440 Version,

always « normal »

position used

In R460,

« normal » or

« redundant »

according to fan

configuration

Never used

Factory Default

Only SAS version

| Document owner: | COE Servers |
| --- | --- |
| Applies to: | R440 / R460 |