Technical Information Document

COE

Servers

| Document Title: | USB RICC Minicom is not working correctly on R620 / R620 E1 |
| --- | --- |
| Document ID: | TID0704161501 |
| Creation Date: | 16 Apr. 07 |
| Modified Date: |  |
| Document Revision: | 0 |
| Product Class: | Servers |
| Product and Version: | R620 & R620 E1 |

Symptoms / Facts

The mouse and keyboard become crazy.

The mouse is moving everywhere on the screen, closing application, right click, left click, double click …

The keyboard is typing alone under application.

Cause

The following USB RICC are faulty:

6929970000 KVM RICC USB

and

8014200000 KVM RICC USB ROHS

Solutions / Fixes

Contact your service representative

Replace the USB RICC by the following reference:

8028000000 KVM RICC USB ROHS FT

See also

TID0704161500 Graphical mode is not working under LINUX operating system

TID0704161502 PS2 RICC Minicom is not working correctly on ServerEngines Pilot Chipset

TID0704161503 How to update RICC Minicom Firmware and KVM Minicom Firmware

| Document owner : | COE Servers |
| --- | --- |
| Applies to : | R620 – R620 E1 |