Technical Information Document

COE

Servers

| Document Title: | PS2 RICC Minicom is not working correctly on ServerEngines Pilot Chipset |
| --- | --- |
| Document ID: | TID0704161502 |
| Creation Date: | 16 Apr. 07 |
| Modified Date: |  |
| Document Revision: | 0 |
| Product Class: | Servers |
| Product and Version: | R460 – R440 – T820 – T860 |

Symptoms / Facts

The mouse and keyboard become crazy.

The mouse is moving everywhere on the screen, closing application, right click, left click, double click …

The keyboard is typing alone under application.

Cause

The following PS2 RICC are faulty:

6929960000 KVM RICC PS2

and

8014190000 KVM RICC PS/2 ROHS

Solutions / Fixes

Flash the Firmware with the version “pro_rem_PS2_M_v1_6_B121_20070401_E.hex”.

See also

TID0704161500 Graphical mode is not working under LINUX operating system

TID0704161501 USB RICC Minicom is not working correctly on 320Fa

TID0704161503 How to update RICC Minicom Firmware and KVM Minicom Firmware

| Document owner: | COE Servers |
| --- | --- |
| Applies to: | R460 – R440 – T820 – T860 |