BIOS for Novascale Universal R421

bought with Redhat/XBAS or without Operationg System.

R421X020      2008-Apr-4

Novascale R421

(X7DBR-i)

Flash BIOS R 2.0a

« R421X020.ROM»

Built with Phoenix BIOS Editor

contains DBR8C037.exe  (12/03/07)

Pour écrire

pas de trou

Release Notes

R421X020.IMG (2008-Apr-4)

This version has been built with the official package DBR8C037.exe

It contains the BIOS Rev. 2.0a  dated from (12/03/2007)

The version provides the possibility to the operator to enter or not the DMI String : system-serial-number.

FB	XAN-SM1-00012

 Reset the DMI strings and set the values of the image overloading by the options of the command phlash16

FB

 Don’t reset the DMI strings, keep the old values overloading them with the options of the command phlash16.

Read the difference /mode=  of the command phlash16.exe

Other difference :

The tools IPNMAC.EXE and IPMICFG.EXE were add to the diskette so that the operator can insert the IP address of the l’AOC-SIMSO[+].

R421X011.IMG

This version has been built with the official package DBR88137.exe

It contains the BIOS Rev. 1.3c dated from (08/30/2007)

Bull modifications

1-New options in command phlash16.exe

/EXIT	Phlash16 does not reboot the system after completing its programming.

2-DMI string "Motherboard version" is forced to:

R421W011 BIOS Rev. 1.3c

R421X011 BIOS Rev. 1.3c

This field can be read from the operating system.

R421X010.IMG

This version has been built with the official package DBR88137.exe

It contains the BIOS Rev. 1.3c dated from (08/30/2007)

BIOS 1.3c release notes

- Fixed some W39V080FAP/FAPZ parts not reliable when updating BIOS via phlash16.

- Supporting Harpertown/Wolfdale-DP processors.

- Fixed Fan-Failure events intermittent triggered in OS.

- a)Supported to access SMBus interface via IPMI, if existed, when fan fails

b)Initializing IPMI if it got reset by BMC command.

c)Set to 115200 baudrate for PepperCon BMC.

- Fixed LSI SAS31601E firmware incorrectly activated when BIOS sets to disable (SAS board)

- Fixed onboard LAN failed to boot if one card with P2P bridge plugged before its parent bridge.

- Add workaround for MCH BSU v1.03 Errata #24.

- Add workaround for ESB2 Errata #100.

- a)Default Date to 07/01/2007 if CMOS low.

b)Updated Intel Xeon micro-code.

- Supported mixed stepping for Clovertown or Woodcrest.

- Support JPE4-managed U riser with one or two PCI-e slots.(X7DBR-8+)

- Added BWA for ESB2 Spec Update Errata #99.

- Fixed Sony SDX-5xx Tape drive not recognized.

- Fixed <ESC>+<!> for F11 hotkey not working via UCR.

- PXE ROM hangs upon first boot after flashing with CMOS cleared.

- Defaulted to Coalesce for PCI-e I/O Performance option.

R421X002.IMG

This version has been built with the official package DBR85297.exe

It contains the BIOS Rev. 1.3b dated from (05/30/2007)