BIOS for Novascale Universal R422E1 or R422E1-INF

bought with Redhat/XBAS or without Operationg System.

R4221X60              November 20, 2008

R422-E1/R422-INF-E1

(X7DWT/X7DWT-INF)

Flash BIOS 7DWTB048.exe

« R4221X60.ROM»

with Logo Bull

Built with Phoenix BIOS Editor 2.2.0.9

Bios Rev. 1.2 X7DWT (2008/11/19)

write enable

no hole

Release notes :

R4221X60.IMG (2008-November-20)

This version has been built with the official package 7DWTB048.exe (2008/11/19)

It contains the BIOS Rev. 1.2 for X7DWT and X7DWT-INF

Bios improvements: To see : ECO_12683.889.1.pdf

- Fix IPMI connection may be broken in system reset (ALL)

- Fix system configuration data is not updated to IPMI if power on system 1 minute after AC cable applied (ALL)

- Fix SLP2 code is conflict with WHEA code (ALL)

- Fix CPU in socket 0 is not selected as BSP sometimes (ALL)

- Update AHCI firmware from 1.17D to 1.20E (ALL)

- Fix USB keyboard cannot work when USB sound device is installed (ALL)

- Fine tune PCI Express init timing (ALL)

- Change Max Payload Size algorithm (ALL)

- Change Crystal Beach to I/OAT2 (ALL)

- Enable HPET by default (ALL)

- Fix CF card bootable issue (ALL)

- Fix SLP2 code is conflict with WHEA code (ALL)

R4221X50.IMG (2008-September-10)

This version has been built with the official package 7DWT8048.exe

But it contains the BIOS Rev 1.1c 7DWT8198.ROM dated from 08/29/2008.

Here are the improvements for X7DWT and X7DWT-INF  (To see : ECO_12552.752.2.pdf)

1-Fix system reboot if the on-board IB is not programmed.

2-Fix CPU in socket 0 is not selected as BSP sometimes.

R4221X41.IMG (2008-August-13)

This version has been built with the package 7DWT8048.exe.

It contains the official BIOS Rev. 1.1b dated from 08/04/2008.

It supports the new INTEL processor stepping E-0.

R4221X40.IMG  (2008-August-4)

This version has been built with the package 7DWT4308.exe  with the version 7DWT7118.rom  in order to solve the problem of boot from PXE when a TESLA cabinet is present.

R4221X30.IMG  (2008-May-23)

This version has been built with the package 7DWT4308.exe.

It contains the BIOS Rev. 1.1a dated from 05/14/08

R4221X23.IMG  (2008-Apr-23)

This version has been built with the package 7DWTC217.exe.

It contains the BIOS Rev. 1.0b dated from 12/31/07

Setup.Advanced.Boot Features.Power Loss Control = Stay Off instead of Last State

R4221X22.IMG

this version R4221X22 is equal to R4221X21 for the BIOS itself and the options of SETUP.

The difference is the possibility provide to the operator to enter or not the DMI String : system-serial-number,.

Exemples :

FB	XAN-SM7-00012

à Reset the DMI strings and set the values of the image overloading by the options of the command phlash16.

FB

àDon’t reset the DMI strings, keep the old values overloading them with the options of the command phlash16.

Read the difference /mode=  of the command phlash16.exe

Other difference :

The tools IPNMAC.EXE and IPMICFG.EXE were add to the diskette so that the operator can insert the IP address of the l’AOC-SIMSO[+].