BIOS for Novascale Universal R425

bought with Redhat/XBAS or without Operating System.

R4250X60               November 20, 2008

Novascale R425

(X7DWA-N)

Flash BIOS 7DWAB048.exe

« R4250X60.ROM»

with Logo Bull

Built with Phoenix BIOS Editor 2.2.0.9

BIOS Rev. 1.2 X7DWA-N (2008/11/19)

Write enable

no hole

Release Notes

R4250W60.IMG (2008-November-20)

This version has been built with the official package 7DWAB048.exe (2008/11/19)

It contains the BIOS Rev. 1.2 for X7DWA-N

Bios improvements: To see : ECO_12683.889.1.pdf

- Fix IPMI connection may be broken in system reset (ALL)

- Fix system configuration data is not updated to IPMI if power on system 1 minute after AC cable applied (ALL)

- Fix SLP2 code is conflict with WHEA code (ALL)

- Fix CPU in socket 0 is not selected as BSP sometimes (ALL)

- Update AHCI firmware from 1.17D to 1.20E (ALL)

- Fix USB keyboard cannot work when USB sound device is installed (ALL)

- Fine tune PCI Express init timing (ALL)

- Change Max Payload Size algorithm (ALL)

- Change Crystal Beach to I/OAT2 (ALL)

- Enable HPET by default (ALL)

- Fix CF card bootable issue (ALL)

- Fix SLP2 code is conflict with WHEA code (ALL)

- Fix CD1 line cannot work (only in X7DWA-N)

R4250X40.IMG (August 14, 2008)

This version has been built with the official package 7DWA8048.EXE

It contains the BIOS Rev. 1.1b  X7DWW-N dated from 08/04/2008

It supports the INTEL Harpertown processor with stepping E-0.

R4250X30.IMG (23 May 2008)

This version has been built with the official package 7DWA4308.exe

It contains the BIOS Rev. 1.1 X7DWW-N  dated from  05/14/2008

To reduce the boot duration the parameter

Setup.Advanced.Boot Features.Quick Boot was set to “ Enabled”.

R4250X23.IMG (28 avril 2008)

This version has been built with the official package 7DWAC217.exe

It contains the BIOS Rev. 1.0b dated from December 31, 2007

To support the SOL (Serial Over LAN) function of the AOC-SIMLP-B+

the  Setup.Advanced.IO Device Configuration.Serial Port B   was set to “Enabled”

Setup.Advanced.Boot Features.Power Loss Control = Stay Off instead of Last State

Support of the RAID HBA AOC-USAS-S8iR in the boot list.

R4250X21.IMG

This version has been built with the official package 7DWAC217.exe

It contains the BIOS Rev. 1.0b dated from December 31, 2007

The difference is the possibility provide to the operator to enter or not the DMI String : system-serial-number,.

Examples :

FB	XAN-SM8-00012

 Reset the DMI strings and set the values of the image overloading by the options of the command phlash16.

FB

 Don’t reset the DMI strings, keep the old values overloading them with the options of the command phlash16.

Read the difference /mode=  of the command phlash16.exe

Other difference :

The tools IPNMAC.EXE and IPMICFG.EXE were add to the diskette so that the operator can insert the IP address of the l’AOC-SIMSO[+].